# CASE STUDY 2: REVIEW 1

Course Code: 23CSE343 | Course Name: Wireless and Mobile Networks

Team No. 5

Team Members: Jai Subiksha T (CB.SC.U4CSE23327) | Mahasri M (CB.SC.U4CSE23335) | Sanjai P G (CB.SC.U4CSE23366)

# Section 1: Demonstrate the behaviour of Mobile IP

## 1.1 Selection of Protocol

We selected Mobile IPv4 for this demonstration. The implementation uses Python and the Scapy library to programmatically craft, transmit, and dissect Mobile IP Registration Request and Reply packets to simulate node mobility across networks.

## 1.2 Brief Explanation and Illustration

Mobile IP allows a Mobile Node (MN) to keep the same IP address (Home Address) even when moving to a different network (Foreign Network). When the MN moves, it receives a temporary Care-of Address (CoA) from a Foreign Agent (FA). The MN registers this CoA with its Home Agent (HA) so the HA can tunnel packets to the new location.

## 1.3 Packet Headers and Formats

A Mobile IP Registration Request includes the following critical fields:
- Type: 1 (Request) / 3 (Reply)
- Lifetime: Registration validity period (e.g., 3600s)
- Home Address: The permanent IP of the MN
- Home Agent Address: The IP of the HA
- Care-of Address: The temporary routing IP
- Identification: A 64-bit timestamp/nonce for security.

## 1.4 Network Design for the Demonstration

We designed two network scenarios using Scapy:

- Network Design 1 (Base): Normal successful registration where MN sends a request via FA to HA, and receives a Code 0 (Success) reply.

- Network Design 2 (Failure): The Foreign Agent (FA) crashes or rejects the request due to lack of resources, sending a Code 66 reply.

## 1.5 Simulation of Network Design along with Mobile IP

Below are the exact execution traces and hex dumps of the crafted packets generated by our Python Scapy script.

============================================================
CASE STUDY 2 - PART 1: MOBILE IP SIMULATION (SCAPY)
============================================================

[1] NETWORK DESIGN 1: Successful Mobile IP Registration
------------------------------------------------------------
Scenario: Mobile Node (MN) moves to a new network and registers its Care-of Address (CoA) with the Home Agent (HA) via the Foreign Agent (FA).

MN Home Address: 192.168.1.50
Home Agent (HA): 192.168.1.1
Foreign Agent (FA): 10.0.0.1
Care-of Address: 10.0.0.50

>>> CRAFTING: Mobile IP Registration Request

[Registration Request - Packet Hierarchy]
###[ IP ]###
  version   = 4
  ihl       = None
  tos       = 0x0
  len       = None
  id        = 1
  flags     = 
  frag      = 0
  ttl       = 64
  proto     = udp
  chksum    = None
  src       = 10.0.0.50
  dst       = 192.168.1.1
  \options   \
###[ UDP ]###
     sport     = 434
     dport     = 434
     len       = None
     chksum    = None
###[ Mobile IP (RFC3344) ]###
        type      = RRQ
###[ Mobile IP Registration Request (RFC3344) ]###
           flags     = 0x0
           lifetime  = 3600
           homeaddr  = 192.168.1.50
           haaddr    = 192.168.1.1
           coaddr    = 10.0.0.50
           id        = 1234567890

[Registration Request - Hex Dump]
0000  45 00 00 34 00 01 00 00 40 11 AE DD 0A 00 00 32  E..4....@......2
0010  C0 A8 01 01 01 B2 01 B2 00 20 47 40 01 00 0E 10  ......... G@....
0020  C0 A8 01 32 C0 A8 01 01 0A 00 00 32 00 00 00 00  ...2.......2....
0030  49 96 02 D2                                      I...

>>> CRAFTING: Mobile IP Registration Reply (Success)

[Registration Reply - Packet Hierarchy]
###[ IP ]###
  version   = 4
  ihl       = None
  tos       = 0x0
  len       = None
  id        = 1
  flags     = 
  frag      = 0
  ttl       = 64
  proto     = udp
  chksum    = None
  src       = 192.168.1.1
  dst       = 10.0.0.50
  \options   \
###[ UDP ]###
     sport     = 434
     dport     = 434
     len       = None
     chksum    = None
###[ Mobile IP (RFC3344) ]###
        type      = RRP
###[ Mobile IP Registration Reply (RFC3344) ]###
           code      = 0
           lifetime  = 3600
           homeaddr  = 192.168.1.50
           haaddr    = 192.168.1.1
           id        = 1234567890

[Registration Reply - Hex Dump]
0000  45 00 00 30 00 01 00 00 40 11 AE E1 C0 A8 01 01  E..0....@.......
0010  0A 00 00 32 01 B2 01 B2 00 1C 4F 7A 03 00 0E 10  ...2......Oz....
0020  C0 A8 01 32 C0 A8 01 01 00 00 00 00 49 96 02 D2  ...2........I...

[+] Network Design 1 Complete: MN successfully registered with HA.


============================================================
[2] NETWORK DESIGN 2: Foreign Agent Failure Case
------------------------------------------------------------
Scenario: The Foreign Agent (FA) crashes or rejects the registration (Code 66 - insufficient resources or FA unreachable). The MN is left isolated.

>>> CRAFTING: Mobile IP Registration Reply (Failure)

[Registration Failure Reply - Packet Hierarchy]
###[ IP ]###
  version   = 4
  ihl       = None
  tos       = 0x0
  len       = None
  id        = 1
  flags     = 
  frag      = 0
  ttl       = 64
  proto     = udp
  chksum    = None
  src       = 192.168.1.1
  dst       = 10.0.0.50
  \options   \
###[ UDP ]###
     sport     = 434
     dport     = 434
     len       = None
     chksum    = None
###[ Mobile IP (RFC3344) ]###
        type      = RRP
###[ Mobile IP Registration Reply (RFC3344) ]###
           code      = 66
           lifetime  = 0
           homeaddr  = 192.168.1.50
           haaddr    = 192.168.1.1
           id        = 1234567891

[Registration Failure - Hex Dump]
0000  45 00 00 30 00 01 00 00 40 11 AE E1 C0 A8 01 01  E..0....@.......
0010  0A 00 00 32 01 B2 01 B2 00 1C 5D 47 03 42 00 00  ...2......]G.B..
0020  C0 A8 01 32 C0 A8 01 01 00 00 00 00 49 96 02 D3  ...2........I...

[-] Network Design 2 Complete: Registration rejected. MN cannot route packets.
============================================================

# Section 2: Demonstration of MANET routing protocol

## 2.1 Selection of MANET Routing Protocol

We selected a TDMA-Based MANET routing architecture. We simulated the network using MATLAB and the Wireless Network Toolbox to measure actual Packet Delivery Ratios (PDR) under various node and link failure conditions.

## 2.2 Brief Explanation and Illustration

A Mobile Ad-Hoc Network (MANET) operates without centralized infrastructure. Nodes act as both end-devices and routers, forwarding packets for each other. In a TDMA-based MANET, nodes use Time Division Multiple Access slots to avoid packet collisions while routing data across the dynamic topology.

## 2.3 Packet Headers and Formats

The packets in this MANET simulation contain:
- MAC Header: Source/Dest MAC, Frame Control, Sequence Control.
- Routing Header: Source IP, Destination IP, TTL (Time-to-Live), Next Hop IP.
- Payload: Application data traffic (100 Mbps On/Off traffic).

## 2.4 Network Design for the Demonstration

We deployed 15 mobile nodes in a linear topology (mimicking the AB-3 corridor from Case Study 1). Node 1 is the Source and Node 15 is the Destination, requiring multi-hop routing.

- Design 1: Normal Base topology.

- Design 2: Link Failure (Node 8 moves out of radio range of Node 7).

- Design 3: Node Failure (Node 8 is powered off / battery dies).

- Design 4: Both Failures.

## 2.5 Simulation of Network Designs

Sequence of Operations:

1. The network is initialized using our core mathematical routing simulation.
2. 15 nodes are placed in AdHoc mode.
3. The simulation is run for all 4 network designs, and the Packet Delivery Ratio (PDR) is extracted.

====================================================
 CASE STUDY 2 - PART 2: MANET ROUTING SIMULATION 
====================================================

[1] Design 1: Normal Base Topology: ROUTE FOUND. PDR = 97.69%
[2] Design 2: Link Failure (Gap > TxRange): ROUTE FAILED. PDR = 0.00%
[3] Design 3: Node 8 Failure: ROUTE FAILED. PDR = 0.00%
[4] Design 4: Both Failures: ROUTE FAILED. PDR = 0.00%